EXPERIMENT 03

TITLE: Creation of Simple Network Topology using Mininet

AIM: To create a simple network topology using an open-source network virtualization tool (Mininet).

OBJECTIVE:

1. To install Mininet

2. To create topology - single switch with 3 hosts

3. To verify connectivity using Linux commands

4. To stop the network

SOFTWARE USED:

Oracle VirtualBox, Ubuntu 22.04, Mininet 2.3.0

THEORY:

Mininet is a network emulator which creates virtual hosts, switches, controllers and links on a single Linux machine. It uses Open vSwitch. Command sudo mn --topo single,3 creates a topology where 3 hosts are connected to 1 central switch.

PROCEDURE & EXECUTION:

1. Install Mininet

sudo apt update

sudo apt install mininet -y

2. Create Topology

mn --version

sudo mn --topo single,3

3. Verify Network

mininet> nodes

available nodes are: h1 h2 h3 s1

mininet> net

h1 h1-eth0:s1-eth1

h2 h2-eth0:s1-eth2

h3 h3-eth0:s1-eth3

mininet> pingall

Results: 0% dropped (6/6 received)

4. Stop Network

mininet> exit

sudo mn -c

Output: Cleanup complete

RESULT:

Successfully created virtual network with 1 switch and 3 hosts and verified communication. pingall showed 0% packet loss.

CONCLUSION:

We learned to create and configure virtual networks using Mininet, an open-source tool, and verified it using Linux networking commands.